# LC-HRMS Scan-to-sequence-list SOP

Prepared by: Maria Cardelino

Last edit: 11/5/2025

Overview

This application automates the process of converting Matrix ID data from sample tube racks into formatted sequence lists for LC-HRMS analysis. The app supports both C18 and HILIC analysis methods and handles up to 4 96-well plates that map to a single 384-well plate format.

This application significantly reduces manual work and potential errors in preparing LC-HRMS sequence lists while ensuring consistent formatting and sample tracking.

Folder structure – essential items

R:/diwalke/LC/Run_Lists/<Study_folder>/

- Samples.xlsx

- QAQCs.xlsx

- YYMMDD_Sequence_Generator_App/

  - App/

    - Mapping/ # contains instructions for generating 384wp positions.

    - server.R # server logic; instructions for actions performed in the app

    - ui.R # ui logic; instructions for how the app looks

    - write_list.R # instructions for writing the sequence file

  - RUN.R # open this in RStudio to run app.

Application Structure

The application is built using R Shiny and consists of four main tabs:

- Upload Plate Scans

- Check Layout

- Connect to Inventory

- Generate Sequence List

Requirements - Software Dependencies:

required_packages <- c(  "openxlsx", "shinyjs", "tools",  "shiny", "readr", "dplyr",   "plotly", "readxl", "shinyFiles", "stringr", "this.path")

Detailed SOP

Prepping app for a new study:

Open “RUN.R” and edit the following lines to match the names of the excel inventory files in the study folder AND the name of the study folder in 1-Raw Files and E:/ drive (complete with date).

Tab 1: Upload Plate Scans

User Actions:

- Upload 1-4 CSV files containing Matrix IDs from plate scans

  - Ex for CLU0116: First two plates are study samples, 3rd plate is QAQC

- Specify plate type for each uploaded file (Study Sample or QAQC)

- Click "Process Files" to begin analysis

Backend Processing:

- Reads CSV files using dynamic header detection

- Maps 96-well positions to 384-well format

- Creates placeholders for missing plates

- Combines data from all plates into unified format

Tab 2: Check Layout

User Actions:

- Review combined data table, verify plate visualizations

Tab 3: Connect to Inventory

User Actions:

- Click "Step 1: Match IDs to inventory"

- Review the matched Sample_IDs in table AND QAQC plate layout on the right.

  - Hover mouse over colored dots to read sample_ID.

- IMPORTANT: Set injection order (by column: A1, C1, E1 or by row: A1, A3, A5…)

- Click “Step 2: Update inventory” (optional)

  - Checks to make sure samples have not been previously analyzed

  - Updates internal inventory (excel workbook included in parent folder) indicating samples as ‘Analyzed’

Backend processing:

- Matches Matrix IDs with sample inventory in .xslx files

- Displays number of samples matched using Matrix IDs

- Pulls sample IDs for each matched sample and adds to Sample_ID column

- QAQC plate visualization with dynamic color-coded sample types, grouped by name.

Tab 4: Generate Sequence List

User Actions:

- Enter run information (see below)

- Click "Update Run Info" ^^^

- Check filepaths and click Confirm Paths

- Click "Generate Sequence List"

- If generating C18 and HILIC at the same time, return to the top and change instrument to “HILIC”, change position if needed, and click “Update Run Info” AGAIN, re-check filepaths, confirm, then “Generate Sequence List”

** Check errors in the Progress/Debug Console.

** Verify output with finish message:

Output Generation:

- Creates CSV file with formatted sequence list including Sample types, File names, Sample IDs, Injection parameters, Method paths, Well positions, File Output Structure.

Run List Documentation

Current studies (post-Otober 2025)

Study affiliation: SWAN (CLU0116)

Major changes:

- Less QAQC samples

- Checks rack size and adjusts length of run list / which QAQC samples to include accordingly.

- New PFAS QAQCs and calibration curves.

*Bold: always runs

Conditions:

- 2 sample racks, each with >60 samples

  - FULL QAQC

- 2 sample racks, second rack < 60 samples

  - 1 less pool (5 instead of 6)

- 1 sample rack only:

  - 3 less pool (3 instead of 6)

  - 1 less PFAS baby cal curve (only need 2 of each calibration point instead of 3)

  - 1 less PFAS QAQC (only need 2 of 3)

CLU0116 RUN LIST:

1 real blank (front and back labeled 'Blank', middle labeled 'Cal\_Std')

1 water method blank (out of 2)

1 NIST (OPTIONAL, out of 1)

1 AMAP (optional, out of 1)

3 PFAS baby cal curve (0.1, 1, 10)

PFAS QAQC (1/3)

HRE Pool 1 and 2 (1)

Study samples

HRE Pool 1 and 2 (2)

only runs if first rack > 60 samples: {

|	Study samples

|	HRE Pool 1 and 2 (3)

}

PFAS QAQC (2/3) - only if there are two sample racks

1 'blank'

BIG CAL CURVE (1-8/8)

1 'blank'

only runs if there are two sample racks: {

|	HRE Pool 1 and 2 (4)

|	Study samples

| 	HRE Pool 1 and 2 (5)

|	only runs if second rack > 60 samples: {

|	|	Study samples

|	|	HRE Pool 1 and 2 (6)

|	}   resume if only one chunk of samples in second half …

|	3 PFAS baby cal curve (0.1, 1, 1)

}  resume if only one sample rack

PFAS QAQC (3/3 or 2/2)

1 water method blank (out of 2)

1 real blank

Old order of injections for CLU0120_MEC: (Before October 2025)

# 1 blank

# 1 water (1 of 2)

# 1 NIST (1 of 1)

# 1 AMAP (1 of 2)

# Pool 1

# Pool 2

# 20 study samples

# Pool 1

# Pool 2

# 20 study samples

# Pool 1

# Pool 2

# 20 study samples

# Pool 1

# Pool 2

# 20 study samples

# Pool 1

# Pool 2

# 1 'blank'

# CAL CURVE (1-8/8)

# 1 'blank'

# Pool 1 (6/10)

# Pool 2

# 20 study samples

# Pool 1

# Pool 2

# 20 study samples

# Pool 1

# Pool 2

# 20 study samples

# Pool 1

# Pool 2

# 20 study samples (160 total)

# Pool 1

# Pool 2

# 1 AMAP (2 of 2)

# MDL (Cal curve)

# Water (2 of 2)

# 1 blank